# Manufacturers meeting protocol 18.07.07

Members present: see list. Eugen Stiebritz (Skyward) and Markus Gründhammer (Wings of Change) were excused.

Meeting Start 10:15, Meeting  End 16:30

DHV Managing director Klaus Tänzler explained how the DHV wants to uphold the trust placed by pilots in the DHV Gütesiegel testing program. The classification declared must match the reality. The DHV is very interested to listen to manufacturers advice in order to prevent “taking a turn for the worse” unintentionally.

DHV Safety officer Karl Slezak reminded us of the positive effects of the previous changes made to airworthiness requirements (LTF) 5 years ago. Unfortunately, at that time, the changes were made a year too late. As a result certain paragliders achieved ratings for classes in which they were not suitable for, and this was later demonstrated in the accident statistics. This time, the DHV wants to act pre-emptively. Advances in airsports equipment development must be matched with the continued development of equipment testing and airworthiness requirements in order that certification remains relevant. There are renewed signs that the current airworthiness tests where paragliders are rated at the top end of a category allows for  a  paraglider classification unsuited for the invisaged pilot group.

Karl Slezak explained the conclusions drawn from the accident statistics, and agreed to send them to the manufacturers per email. The DHV Technik manager Hannes Weininger explained how the technical department will react to the new developments.

The following text is concerned with points in the proposed changes to airworthiness requirements which lead to debate and conclusion.

# Collapses

It was discussed in how far the new DHV proposals for testing the form of a collapse do justice to collapses experienced in practice. The manufacturers warned that this new form of testing must be clearly defined.

It is proposed in addition to the existing airworthiness requirements, to introduce collapse tests with a 60° folding line for 50-60% of the paragliders span.

The DHV agrees to postpone decision until this proposal has been tested extensively. Manufacturers factory pilots will be invited to attend testing.

The DHV is to inform all manufacturers of the investigation test results of their own products.

# Spiral dives

Manufacturers reported problems with large variations in results for this test manoeuvre. In addition, it was suggested to increase the limit proposed by the DHV from 10 m/s to 12 m/s.

The DHV agreed to investigate methods for minimising measurement variations.

The DHV will postpone until then the decision on the limit proposal.

# Harness side protection

Doubts were voiced, whether the protection offered by a side protector of 2,8 cm thickness is sufficient to warrant inclusion in the airworthiness requirements. Pilots should be able to decide for themselves whether they want to have side protection in their harnesses.

The DHV agrees to send manufacturers the results of side protector testing, and considered rewording the proposal, that the use of side protectors must only be possible in all harnesses.

# Harness load testing

An increase of 15 g for the load test, where a reserve parachute is attached to two points on a harness, was discussed.

The DHV agrees to make sketches of the attachment points of the protector dummy to clarify the new proposed protector test.

Manufacturers were of the opinion that increasing the load test for a single point of attachment on a harness from 9 g to 10 g would create problems.

The DHV agrees not to impliment this proposal.

# Paraglider line strength

It is feared that the DHV proposal would result in a reduction of 0,5 in glide angle if implimented, and that the current accident statistics do not justify a harder test.

It was proposed that the current limits be maintained, and that the customer should be informed of minimum breaking strengths.

The DHV agrees to impliment this proposal.

Paraglider brake lines

It was unanomously agreed that a permanent marking of the brake line range is necessary on all paragliders.

Reserve parachute sink rates / opening times

It was agreed that the proposed reduction in sink rate limit increases the opening time of a reserve. In addition, testing procedures for analysing pendulum stability while sinking were requested to be considered.

The DHV agrees to change its proposal, such that the current limits are maintained, and that test reports are to include actual measured sink rates for different payloads.

18.7.2007

Petra Aichele